ALRE

EBR2006N…             EBR2008N…

Combination Gas Valves  EBR2006N...

EBR2008N...

The combination gas valves type EBR... have been developed for use in gas-fired domestic central heating boilers and water heating appliances with automatic ignition systems. The controls are also suited for use on a wide variety of gas-fired appliances

Use

• Compact design suitable for installation in small modern boilers and heaters

• Specially designed for gas appliance with DBI system to light the main burner

• Servo pressure regulator ensures stable outlet pressure

• Pressure feedback ensures constant burner pressure

Functional options  EBR2006N...    • Fast and slow opening version for on/off control

• Adjustable slow opening to ensure smooth light-up of burner

EBR2008N...    • Modulating control for more comfort and energy savings

Mechanical options • 2 shutoff valves

• Servo pressure regulator

• Inlet/outlet pressure test points

• All adjustments are accessible from the top of valve

• A fine-mesh screen is integrated at the inlet side

CC1N2008cn                                                                         ALRE

05.05.2009                                                                   HVAC Products

Technical data

General unit data    Models                               refer to《Type code》

Mounting position                      vertical or horizontal±5°

Type of gas                           CITY-GAS、LNG、LPG

Gas inlet pressure                     max. 60mbar

Operating voltage tolerance             gas valve operates correctly between

85% and 110% of the rated voltage

Degree of protection                    IP44 with optional connector

Dimensions                           refer to《Dimensions》

Inlet filter                              no. 100 fine mesh

Pipe connection inlet and outlet          refer to《Type code》

Pressure feedback connection           6.5mm outside dia. for tube

connection

Regulation capacity                     min. 0.31m3/h air

Outlet pressure range for ON/OFF        1...37mbar

Controller

Outlet pressure range for modulating      1...37mbar

Controller

Valve class                     EN126

-1st safety shutoff valve             class B

-2nd safety shutoff valve                 class D, class J

Class of regulator                       EN126

- EBR2006N...                           class C

- EBR2008N...                           class C

Closing time of safety shutoff valves       within 1S

Weight

-EBR2006N...                            approx. 870g

-EBR2008N...                            approx. 980g

Electrical data      Power consumption and current

| Type | SV1 shutoff valve |  |  | SV2 shutoff valve |  |  | SV1 and SV2 shutoff valve |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Voltage | Power | Current | Voltage | Power | Current | Voltage | Power | Current |
| EBRxx0 | … | … | … | … | … | … | 230VAC | 13VA | 60mA |
| EBRxx1 | … | … | … | … | … | … | 24VAC | 13VA | 570mA |
| EBRxx2 | 145VRAC | 8.2VA | 60mA | 85VRAC | 4.8VA | 60mA | 230VRAC | 13VA | 60mA |
| EBRxx3 | 15.4VRAC | 8.2VA | 570mA | 8.6VRAC | 4.8VA | 570mA | 24VRAC | 13VA | 570mA |
| EBRxx4 | 230VRAC | 8.2VA | 38mA | 230VRAC | 4.8VA | 22mA | … | … | … |
| EBRxx5 | 24VRAC | 8.7VA | 380mA | 24VRAC | 4.3VA | 190mA | … | … | … |
| EBRxx6 | 15.7VDC | 8.5VA | 540mA | 8.3VDC | 4.5VA | 540mA | 24VDC | 13VA | 540mA |
| EBRxx7 | 24VDC | 8.7VA | 360mA | 24VDC | 4.3VA | 180mA | … | … | … |
| EBRxx8 | … | … | … | … | … | … | 120VAC | 13VA | 120mA |

2/11

ALRE                                                                         CC1N2008cn

HVAC Products                                                                   05.05.2009

Technical data

Only EBR2008N...       Electrical modulating pressure regulator

| Operating voltage | Current | Resistance |
| --- | --- | --- |
| Max. 13.2V | Max. 165mA | 80Ω |
| Max. 24.8V | Max. 310mA | 80Ω |

Note             The coils for the safety shutoff function of the EBR... gas valve are DC type.

Pin connections      2 types of terminal arrangements are available for using different types of cable

connectors

Variant 0

Full wave rectifier is integrated in the ignition equipment.

Variant 1

Note：

The coils for safety shutoff function of EBR... gas valve are DC type

3/11

ALRE                                                                         CC1N2008cn

HVAC Products                                                                   05.05.2009

Function

Modulating control     Modulating coil：13.2V/165mA

Characteristics of

EBR2008N...

Modulating coil：9V/310mA

4/11

ALRE                                                                         CC1N2008cn

HVAC Products                                                                   05.05.2009

Function

Capacity in m3/h air at     Solenoid valve class                           B+D(J)

pressure differential       Flow rate at a pressure drop of 5mbar            4.3m3/h air

Flow chart of classes B and C

EBR2008N-G3/4...

EBR2008N-G1/2...

EBR2006N...

5/11

ALRE                                                                         CC1N2008cn

HVAC Products                                                                   05.05.2009

Function

Legend    1  1st shutoff valve(class B)              8   Servo valve

2  2nd shutoff valve                     9   Diaphragm for slow opening

3  cap                                 10  Modulating coil

4  Setting screw minimum outlet pressure  11  Main diaphragm

5  Servo membrane                     12  Adjusting screw for slow opening

6  Setting screw for servo pressure        13  Inlet pressure

regulator                             14  Outlet pressure

7  Setting screw maximum outlet pressure  15  Combustion chamber feedback pressure

16  Filter

6/11

ALRE                                                                         CC1N2008cn

HVAC Products                                                                   05.05.2009

Warning notes

To avoid injury to persons, damage to property or the environment, following

warning notes must be observed！

Do not open, interfere with or modify the valve!

• All activities (mounting, installation, service work, etc.) must be performed by qualified staff

• Before performing any wiring changes in the connection area of the EBR…, completely isolate the unit from the main supply (all-polar disconnection)

• Ensure protection against electric shock hazard by providing adequate protection for the connection terminals.

• Check to ensure that wiring is in an orderly state

• Fall or shock can adversely affect the safety functions. Such valves must not be put into operation even if the unit does not exhibit any damage

Mounting notes

• Ensure that the relevant national safety regulations are complied with

Outlet pressure of servo pressure regulator –on/off control of EBR2006N…

• Check the inlet and outlet pressure using the pressure test points

provided

• Remove the protective cap,

- turn the screw (white) clockwise to increase the outlet pressure

- turn the screw (white) counterclockwise to decrease the outlet

pressure

• Ensure that the outlet pressure is correctly adjusted before replacing

the protective cap

Outlet pressure of servo pressure regulator – modulating control of EBR2008N

• Check the inlet and outlet pressure using the pressure test points

provided

Remove the protective cap (yellow)

If the inlet pressure is the normal pressure, then add the maximum

Current to the modulating coil.

To set the maximum outlet pressure:

- turn the screw (brass) clockwise to increase the output pressure

- turn the screw (brass) counterclockwise to decrease the output

pressure

• Ensure that the maximum outlet pressure is correctly adjusted with

change of current to the modulating coil.

To set the minimum outlet pressure:

If the inlet pressure is the normal pressure, disconnect the terminals

of the modulating coil (modulating current= 0 mA)

- turn the red screw clockwise to increase the outlet pressure

- turn the red screw counterclockwise to decrease the outlet pressure

• Ensure that the outlet pressure is correctly adjusted, check minimum and maximum setting several times, then tighten the protective cap for protection

7/11

ALRE                                                                         CC1N2008cn

HVAC Products                                                                   05.05.2009

Installation notes

Main gas connection  • To prevent distortion and/or damage of the external thread, take care not to

tighten the pipe fitting too far

• Ensure that the gasket is properly placed in the right position

• Ensure that the gas flow is in the same direction as the arrow on the valve body

Pressure test points  • The valve is provided with an inlet and outlet pressure test point

• When checking the pressure, undo the screw one half turn and slip the tube

over the nipple

Note            Make sure the screw is retightened after making the test.

Electrical connections

Warning          Switch off power supply before making the electrical connections. Wiring must

be in accordance with local regulations. Follow the instructions supplied by the

manufacture.

• Install power in accordance with the required pin connection (refer to Function)

• When making connections to the terminals of the valve, use wires and  connectors which are suited for temperature up to 105℃

Checkout and     • After each adjustment, put the control into operation, run it through several com-

Installation            plete cycles and check to ensure that all burner components function correctly

Commissioning notes

• Prior to commissioning the plant, check to ensure wiring is in an orderly state

Service notes

• Each time a unit had been replaced, check to ensure that wiring is in an orderly

state

• Check wiring and all safety functions each time a unit has been replaced

Important         Follow the appliance manufacturer’s service and maintenance instructions.

Gas leakage test   • The gas valves are factory-tested for gas leakage. Only the main burner

connection needs to be checked for gas leakage

8/11

ALRE                                                                         CC1N2008cn

HVAC Products                                                                   05.05.2009

Disposal

The unit contains electric components and must not be disposed of together with

Household waste.

Local and currently valid legislation must be observed.

Accessories

Ignition Equipment, combined with connecting cable

The PEC… consists of a cable for connection to the safety

shutoff valves of the EBR… gas valves and electronic         PEC…

ignition equipment for use on gas boilers with signal- or

double-pole ignition.

Note                      Ignition Equipment has two types: PEC1 is used for gas valve with AC type. PEC2 is used for gas valve with DC type.

Condensation, formation of ice and ingress of water are not permitted！

Life cycle          • Combination Gas Valves type EBR2006N... / EBR2008N... has a designed

lifetime* of 250,000 burner startup cycles which, under normal operating

conditions in heating mode, correspond to 10 years of usage.

The designed lifetime is based on use of the Combination Gas Valves

According to the manufacturer’s operating instructions.

After reaching the designed lifetime in terms of the number of burner startup

cycles, or the respective time of usage, the Combination Gas Valves is to be

replaced by authorized personnel.

* The designed lifetime is not the warranty time specified in the Terms of

Delivery.

9/11

ALRE                                                                         CC1N2008cn

HVAC Products                                                                   05.05.2009

Dimensions

Dimensions in mm

EBR2008N...

10/11

ALRE                                                                         CC1N2008cn

HVAC Products                                                                   05.05.2009

Dimensions

Dimensions in mm

EBR2006N...

11/11

ALRE                                                                         CC1N2008cn

HVAC Products                                                                   05.05.2009